LAB: GPIO Digital InOut(eval board)

LAB: GPIO Digital InOut

Date: 2025-09-16

Author/Partner: 22201042-Dongjun,Lee

Github: https://github.com/henny041520-commits/EC-DJLee-042/tree/main

Demo Video:

LAB_GPIO_DIO_LED_Photosensor_22201042 :

https://youtube.com/shorts/xrf6yfmJF2U?feature=share

LAB_GPIO_DIO_LED_Button_22201042 :

https://youtube.com/shorts/p9E1w-iWDRE?feature=share

LAB_GPIO_DIO_multiLED :

https://youtube.com/shorts/MM9G_ndksnY?feature=share

Introduction

In this lab, you are required to create a simple program that toggle multiple LEDs with a push-button input. Create HAL drivers for GPIO digital in and out control and use your library.

Requirement

Write a list of HW/SW requirement

Hardware

- MCU

  - NUCLEO-F411RE

  - Eval Board

- Sensor

  - Photodetector

- Actuator/Display

  - LED

Software

- -PlatformIO, CMSIS, EC_HAL library

Problem 1 :  Create EC_HAL library

Procedure

Library Header Files

Create the library directory \repos\EC\include

Download necessary library files:

ecRCC2.h, ecRCC2.c, ecPinNames.h, ecPinNames.c, ecGPIO2.c, ecGPIO2.h

Description with Code

ecGPIO2.h

Additional Definition for lowspeed~nopud for coding convenience

Discussion

- Find out a typical solution for software debouncing and hardware debouncing.

  - A typical solution for software debouncing is to use time-based filtering or a state change detection

    - Time-based filtering (delay method): After detecting a button press, the program waits for a short delay (about 10-20ms) before confirming the input.

    - State-change detection (edge detection): As used in the LED toggle code, the program only reacts when input changes from 0 -> 1 (rising edge) or 1->0(falling edge). This ensures that the LED toggles once per press

  - A typical solution for hardware debouncing is to use an RC(resistor-capacitor) filter or Schmitt Trigger circuit

    - The RC network smooths out the rapid on/off transitions caused by switch bounce.

    - A Schmitt Trigger provides hysteresis and ensures a clean digital signal at the MCU input.

- What method of debouncing did this NUCLEO board use for the push-button(B1)?

  - The NUCLEO board does not implement hardware debouncing for the B1 push-button. It simply connects the button to the MCU pin with pull-up or pull-down resistor. There for, debouncing must be handled in software. There for, I chose to use ‘State-change detection’ method.

Problem 2: Toggle a single LED with Digital Sensor(Photodetector)

Procedure
1. Create a new project under the directory \repos\EC\lab\

- The project name is “LAB_GPIO_DIO_LED_Photosensor”.

- Name the source file as “LAB_GPIO_DIO_LED_Photosensor.c”

2. Include your library ecGPIO2.h, ecGPIO2.c in \repos\EC\include\.

3. Toggle the LED by covering the photodetector sensor.

- Dark (LED ON), Bright (LED OFF) and repeat

Configuration

| Digital Sensor(Photodectector) | LED |
| --- | --- |
| Digital in | Digital OUT |
| GPIOA, Pin 0 | GPIOC, Pin 3 |
| PULL-UP | Open-Drain, Pull-up, Medium Speed |

Circuit/Wiring Diagram

External circuit diagram that connects MCU pins to peripherals(sensor/actuator)

Algorithm

Mealy FSM Table

| Present State | Next State (X=Dark=0) | Next State (X=Bright=1 | Output when X=0 (Dark) | Output when X=1 (Bright) |
| --- | --- | --- | --- | --- |
| S0 (LED=OFF) | S1 | S0 | LED=ON | LED=OFF |
| S1 (LED=ON) | S1 | S0 | LED=ON | LED=OFF |

Flowchart

Description with Code

-Lab source codehttps://github.com/henny041520-commits/EC-DJLee-042/blob/main/lab/LAB_GPIO_DIO_LED_Photosensor/LAB_GPIO_DIO_LED_Photosensor.c

Explain your source code with necessary comments

-Description 1

Setup

void setup(void)

{

RCC_HSI_init();                     <- enable 16MHz HSI clock

GPIO_init(button_pin, INPUT);       <- PA0: photosensor input

GPIO_pupd(button_pin, pullup);      <- enable internal pull-up

GPIO_init(LED_pin, OUTPUT);         <- PB12: LED output

GPIO_otype(LED_pin, opendrain);     <- output type = Open-Drain

GPIO_pupd(LED_pin, pullup);         <- line held HIGH when not driven

GPIO_ospeed(LED_pin, mediumspeed);  <- output slew = Medium

}

-Description 2

int main(void) {

setup();

while(1){

if(GPIO_read(button_pin) == 0)  <- GPIO_read(button_pin) = 0(dark)

GPIO_read(button_pin) = 1 (Bright)

GPIO_write(LED_pin, 1);     <- Dark -> LED ON

else

GPIO_write(LED_pin, 0);      <- Bright -> LED OFF

}

}

Results and Analysis

Results

| Results | Analysis |
| --- | --- |
|  | Bright (GPIO_read(button_pin) == 1) LED OFF |
|  | Dark (GPIO_read(button_pin) == o) LED ON |
|  | Bright (GPIO_read(button_pin) == 1) LED OFF |

Demo Video

https://youtube.com/shorts/xrf6yfmJF2U?feature=share

Analysis

- Work as intended

  - Dark -> LED ON , Bright -> LED OFF, behavior is consistent across repeated trials

- Fast response

  - Simple polling loop; no perceptible latency

- Edge flicker

  - At threshold lighting, minor flicker appeared -> It can be solved by mitigating with 10-20ms software debouncing.

Reference

STMicroelectronics, RM0383 — STM32F411xC/E Reference Manual

https://www.st.com/resource/en/reference_manual/rm0383-stm32f411xce-advanced-armbased-32bit-mcus-stmicroelectronics.pdf

STMicroelectronics, UM1724 — STM32 Nucleo-64 Boards User Manual

https://www.st.com/resource/en/user_manual/um1724-stm32-nucleo64-boards-mb1136-stmicroelectronics.pdf

안경잡이 개발자, 아두이노(Arduino) 빛 감지 센서(Photo Resistor)사용해보기

https://blog.naver.com/ndb796/221257578214

Problem 3: Toggle a single LED with a Button Procedure

1. Create a new project under the directory \repos\EC\lab\

- The project name is “LAB_GPIO_DIO_LED_Button”.

- Name the source file as “LAB_GPIO_DIO_LED_Button.c”

2. Include your library ecGPIO2.h, ecGPIO2.c in \repos\EC\include\.

3. Toggle the LED by pushing the button.

- Push button (LED ON), Push Button (LED OFF) and repeat

Configuration

| Button (B1) | LED |
| --- | --- |
| Digital in | Digital OUT |
| GPIOA, Pin 4 | GPIOB, Pin 12 |
| PULL-UP | Open-Drain, Pull-up, Medium Speed |

Circuit/Wiring Diagram

External circuit diagram that connects MCU pins to peripherals(sensor/actuator)

Algorithm

Mealy FSM Table

| Present State | Next State (Event=Rise 0→1) | Next State (Else: 1→0 or Hold) | Output when Event=Rise | Output otherwise |
| --- | --- | --- | --- | --- |
| S0 (LED=OFF) | S1 | S0 | LED=ON | LED=OFF |
| S1 (LED=ON) | S0 | S1 | LED=OFF | LED=ON |

Flowchart

Description with Code

-Lab source code https://github.com/henny041520-commits/EC-DJLee-042/blob/main/lab/LAB_GPIO_DIO_LED_Button/LAB_GPIO_DIO_LED_Button.c

Explain your source code with necessary comments

-Description 1

Setup

void setup(void) {

RCC_HSI_init();                 <- enable 16MHz HSI clock

GPIO_init(BUTTON_PIN, INPUT);   <- PA4: push button input (active-low)

GPIO_pupd(BUTTON_PIN, pullup);  <- enable internal pull-up

GPIO_init(LED_PIN, OUTPUT);     <- PB12 as digital OUTPUT

GPIO_otype(LED_PIN, opendrain); <- output type = Open-Drain

GPIO_pupd(LED_PIN, pullup       <- line held HIGH when not driven

GPIO_ospeed(LED_PIN, mediumspeed);  <- output slew = Medium

}

void setup(void)

{

RCC_HSI_init();

GPIO_init(button_pin, INPUT);

GPIO_init(LED_pin, OUTPUT);   <- PB12: LED output

}

-Description 2

int main(void){

setup();

int buttonState = 1;              <- start released (1)

int prevState   = 1;              <- previous input

int count       = 0;              <- toggle counter (odd=ON, even=OFF)

while(1){

buttonState = GPIO_read(BUTTON_PIN) <- normalize: 0 pressed

1 released

rising edge? pressed(0) -> released(1)

if(prevState == 0 && buttonState == 1)

{

count++;                   <- trigger toggle

}

if(count % 2 == 1)

GPIO_write(LED_PIN, 1);     <- LED ON   (odd)

else

GPIO_write(LED_PIN, 0);     <- LED OFF  (even)

prevState = buttonState;        <- update history

}}

Results and Analysis

Results

| Results | Analysis |
| --- | --- |
|  | 1.Push 2. buttonState = 0; prevState = 1; 3. prevState = buttonState; buttonState = 0; prevState = 0; |

| Results | Analysis |
| --- | --- |
|  | 1.Release 2. buttonState = 1; prevState = 0; 3. (prevState == 0 && buttonState == 1) Count++ (0->1) 4. (count % 2 == 1) LED ON 5. prevState = buttonState; buttonState = 1; prevState = 1; |

| Results | Analysis |
| --- | --- |
|  | 1.Push 2. buttonState = 0; prevState = 1; 3. prevState = buttonState; buttonState = 0; prevState = 0; |

| Results | Analysis |
| --- | --- |
|  | 1.Release 2. buttonState = 1; prevState = 0; 3. (prevState == 0 && buttonState == 1) Count++ (1->2) 4. (count % 2 == 1) else LED OFF 5. prevState = buttonState; buttonState = 1; prevState = 1; |

Demo Video

https://youtube.com/shorts/p9E1w-iWDRE

Analysis

- Work as intended

  - Each release (rising edge 0->1) toggles the LED once

  - Holding the button does not change the state

  - Across repeated trials at different speeds, the behavior remained consistent

- Fast response

  - Simple polling loop; no perceptible latency for human interaction

Reference

STMicroelectronics, RM0383 — STM32F411xC/E Reference Manual

https://www.st.com/resource/en/reference_manual/rm0383-stm32f411xce-advanced-armbased-32bit-mcus-stmicroelectronics.pdf

STMicroelectronics, UM1724 — STM32 Nucleo-64 Boards User Manual

https://www.st.com/resource/en/user_manual/um1724-stm32-nucleo64-boards-mb1136-stmicroelectronics.pdf

CODINGRUN, 아두이노 예제2. 스위치로 LED 켜기 끄기

https://codingrun.com/101

ARUINODOCS, State Change Detection (Edge Detection) for pushbuttons

https://docs.arduino.cc/built-in-examples/digital/StateChangeDetection/

Problem 4: Toggle multiple LEDs with a button

Procedure
1. Create a new project under the directory \repos\EC\lab\

- The project name is “LAB_GPIO_DIO_multiLED”.

- Name the source file as “LAB_GPIO_DIO_multiLED.c”

2. Include your library ecGPIO2.h, ecGPIO2.c in \repos\EC\include\.

3. Connect 4 LEDs externally with necessary load resistors.

- As Button B1 is Pressed, light one LED at a time, in sequence.

- Example: LED0--> LED1--> …LED3--> …LED0….

Configuration

| Button | LED |
| --- | --- |
| Digital in | Digital OUT |
| GPIOA, Pin 4 | PB12,PB13,PB14,PB15 |
| PULL-UP | Push-Pull, Pull-up, Medium Speed |

Circuit/Wiring Diagram

External circuit diagram that connects MCU pins to peripherals(sensor/actuator)

Algorithm

Mealy FSM Table

| Present State | Next State on Rise (0→1) | Next State on Else | Output |
| --- | --- | --- | --- |
| S0 (ALL OFF) | S1 | S0 | 0000 |
| S1 (LED2 ON) | S2 | S1 | 1000 |
| S2 (LED3 ON) | S3 | S2 | 0100 |
| S3 (LED4 ON) | S4 | S3 | 0010 |
| S4 (LED5 ON) | S1 | S4 | 0001 |

Flowchart

Description with Code

-Lab source codehttps://github.com/henny041520-commits/EC-DJLee-042/blob/main/lab/LAB_GPIO_DIO_multiLED/LAB_GPIO_DIO_multiLED.c

Explain your source code with necessary comments

-Description 1

Setup:

void setup(void) {

RCC_HSI_init();                    <- enable 16MHz HSI clock

Button B1 (PA4): digital input with pull-up → active-low (pressed=0)

GPIO_init(BUTTON_PIN, INPUT);

GPIO_pupd(BUTTON_PIN, pullup);

LEDs (PB12~PB15): digital outputs (push-pull, medium speed)

GPIO_init(LED_PIN2, OUTPUT);

GPIO_init(LED_PIN3, OUTPUT);

GPIO_init(LED_PIN4, OUTPUT);

GPIO_init(LED_PIN5, OUTPUT);

GPIO_otype(LED_PIN2, pushpull);

GPIO_otype(LED_PIN3, pushpull);

GPIO_otype(LED_PIN4, pushpull);

GPIO_otype(LED_PIN5, pushpull);

GPIO_ospeed(LED_PIN2, mediumspeed);

GPIO_ospeed(LED_PIN3, mediumspeed);

GPIO_ospeed(LED_PIN4, mediumspeed);

GPIO_ospeed(LED_PIN5, mediumspeed);

LEDs (PB12~PB15): digital outputs with pull-up

GPIO_pupd(LED_PIN2, pullup);

GPIO_pupd(LED_PIN3, pullup);

GPIO_pupd(LED_PIN4, pullup);

GPIO_pupd(LED_PIN5, pullup);}

-Description 2

int main(void) {

setup();

int buttonState = 1;       <-start released

int prevState   = 1;          <- previous input

int count       = 0;       <- toggle counter

)

while(1){

normalize: 0 pressed 1 released

buttonState = GPIO_read(BUTTON_PIN) ? 1 : 0;

rising edge? pressed(0) -> released(1) → advance step

if(prevState == 0 && buttonState == 1){

count++; }

one-hot selection: exactly one LED ON

switch(count){

case 1:

GPIO_write(LED_PIN2, 1);       <- LED_PIN2 ON

break;

case 2:

GPIO_write(LED_PIN2, 0);       <- LED_PIN2 OFF

GPIO_write(LED_PIN3, 1);       <- LED_PIN3 ON

break;

case 3:

GPIO_write(LED_PIN3, 0);       <- LED_PIN3 OFF

GPIO_write(LED_PIN4, 1);       <- LED_PIN4 ON

break;

case 4:

GPIO_write(LED_PIN4, 0);       <- LED_PIN4 OFF

GPIO_write(LED_PIN5, 1);       <- LED_PIN5 ON

break;

case 5:

GPIO_write(LED_PIN5, 0);    <- LED_PIN5 OFF

count = 1;                  <- wrap back to step 1(LED_PIN2 ON)

break;}

prevState = buttonState; }}     <- update history

Results and Analysis

Results

| Results | Analysis |
| --- | --- |
|  | 1.Push 2. buttonState = 0; prevState = 1; 3. prevState = buttonState; buttonState = 0; prevState = 0; |

| Results | Analysis |
| --- | --- |
|  | 1.Release 2. buttonState = 1; prevState = 0; 3. (prevState == 0 && buttonState == 1) Count++ (0->1) 4. switch(count) -> case 1 LED_PIN2 ON 5. prevState = buttonState; buttonState = 1; prevState = 1; |

| Results | Analysis |
| --- | --- |
|  | 1.Push 2. buttonState = 0; prevState = 1; 3. prevState = buttonState; buttonState = 0; prevState = 0; |

| Results | Analysis |
| --- | --- |
|  | 1.Release 2. buttonState = 1; prevState = 0; 3. (prevState == 0 && buttonState == 1) Count++ (1->2) 4. switch(count) -> case 2 LED_PIN2 OFF LED_PIN3 ON 5. prevState = buttonState; buttonState = 1; prevState = 1; |

| Results | Analysis |
| --- | --- |
|  | 1.Push 2. buttonState = 0; prevState = 1; 3. prevState = buttonState; buttonState = 0; prevState = 0; |

| Results | Analysis |
| --- | --- |
|  | 1.Release 2. buttonState = 1; prevState = 0; 3. (prevState == 0 && buttonState == 1) Count++ (2->3) 4. switch(count) -> case 3 LED_PIN3 OFF LED_PIN4 ON 5. prevState = buttonState; buttonState = 1; prevState = 1; |

| Results | Analysis |
| --- | --- |
|  | 1.Push 2. buttonState = 0; prevState = 1; 3. prevState = buttonState; buttonState = 0; prevState = 0; |

| Results | Analysis |
| --- | --- |
|  | 1.Release 2. buttonState = 1; prevState = 0; 3. (prevState == 0 && buttonState == 1) Count++ (3->4) 4. switch(count) -> case 4 LED_PIN4 OFF LED_PIN5 ON 5. prevState = buttonState; buttonState = 1; prevState = 1; |

| Results | Analysis |
| --- | --- |
|  | 1.Push 2. buttonState = 0; prevState = 1; 3. prevState = buttonState; buttonState = 0; prevState = 0; |

| Results | Analysis |
| --- | --- |
|  | 1.Release 2. buttonState = 1; prevState = 0; 3. (prevState == 0 && buttonState == 1) Count++ (4->5) 4. switch(count) -> case 5 LED_PIN5 OFF Count=1 -> case 1 LED_PIN2 ON 5. prevState = buttonState; buttonState = 1; prevState = 1; |

Demo Video

https://youtube.com/shorts/MM9G_ndksnY?feature=share

Analysis

- Work as intended

  - Each rising edge (0→1) of B1 advances exactly one step: LED12 → LED 13 → LED 14 → LED 15 → wrap. Only one LED is ON at any time (one-hot), matching the FSM table.

- Consistency across speeds

  - Even with varying actuation speeds, repeated tests produced consistent results (not moving steps while holding).

- Next steps

  - The code was messy, but since this lab only required verifying a simple outcome, it wasn’t an issue. However, we should identify ways to improve in preparation for future labs.

Reference

STMicroelectronics, RM0383 — STM32F411xC/E Reference Manual

https://www.st.com/resource/en/reference_manual/rm0383-stm32f411xce-advanced-armbased-32bit-mcus-stmicroelectronics.pdf

STMicroelectronics, UM1724 — STM32 Nucleo-64 Boards User Manual

https://www.st.com/resource/en/user_manual/um1724-stm32-nucleo64-boards-mb1136-stmicroelectronics.pdf

CODINGRUN, 아두이노 예제2. 스위치로 LED 켜기 끄기

https://codingrun.com/101

ARUINODOCS, State Change Detection (Edge Detection) for pushbuttons

https://docs.arduino.cc/built-in-examples/digital/StateChangeDetection/